Electrically Driven Hole Spin Resonance Detected with Charge Sensor in a Planar Si CMOS Structure

Aaquib Shamim1, Scott. D. Liles1, Joe Hillier1, Issac Vorreiter1, Fay. E. Hudson2,3, Wee Han Lim2,3, Andrew. S. Dzurak2,3, Alexander. R. Hamilton1

1School of Physics, University of New South Wales, Sydney NSW 2052, Australia

2School of Electrical Engineering and Telecommunications, The University of New South Wales, Sydney NSW 2052, Australia

3Diraq, Sydney, NSW 2052, Australia

Hole-spin qubits based on Si CMOS[1] devices have garnered attention due to their intrinsic spin-orbit interaction (SOI) [2], weak hyperfine interaction, and anisotropic g-tensor[3] [4]. The SOI allows all electrical control of qubits via electric dipole spin resonance (EDSR)[5]which removes the need for the micro-magnets or electric spin resonance strip (ESR) lines making devices less bulky. The weak hyperfine interaction increases the coherence times. The planar Si CMOS structure is industry compatible and combined with individual spin addressability via EDSR, is apt for scaling up to a larger number of qubits. This integration has not yet been shown for a known number of holes in Silicon CMOS.

We studied a hole double quantum dot (DQD) operating in the Pauli spin blocked (2,8) → (1,9) charge transition regime. We were able to operate it as a singlet (S)-triplet (To) qubit [6]. Characterizing the system in singlet-triplet basis provided insights into energy level diagram, position in detuning where Zeeman energy dominates over exchange energy and enabled determination of the g-factors of the dots. We performed microwave driven EDSR of the spins and coherently rotate the spins with a Rabi frequency up to 200 MHz. We also studied the in-plane g-factor anisotropy which varies by 100%. The result demonstrates the capability of industry compatible Si CMOS structure for operating hole spin qubits and allowing local EDSR spin control leading to rapidly driven spin qubits.

[1]	R. Maurand et al., “A CMOS silicon spin qubit,” Nat Commun, vol. 7, Nov. 2016, doi: 10.1038/ncomms13575.

[2]	Roland. Winkler, Spin-orbit coupling effects in two-dimensional electron and hole systems. Springer, 2003.

[3]	A. Crippa et al., “Electrical Spin Driving by g -Matrix Modulation in Spin-Orbit Qubits,” Phys Rev Lett, vol. 120, no. 13, pp. 1–5, 2018, doi: 10.1103/PhysRevLett.120.137702.

[4]	S. D. Liles et al., “Electrical control of the g tensor of the first hole in a silicon MOS quantum dot,” Phys Rev B, vol. 104, no. 23, p. 235303, Dec. 2021, doi: 10.1103/PhysRevB.104.235303.

[5]	D. V. Bulaev and D. Loss, “Electric Dipole Spin Resonance for Heavy Holes in Quantum Dots,” Phys Rev Lett, vol. 98, no. 9, p. 097202, Feb. 2007, doi: 10.1103/PhysRevLett.98.097202.

[6]	S. D. Liles et al., “A singlet-triplet hole-spin qubit in MOS silicon,” Nat Commun, vol. 15, no. 1, p. 7690, Sep. 2024, doi: 10.1038/s41467-024-51902-9.