Large-scale extraction of quantum dots capacitances from industrial FD-SOI spin-qubits devices

Sam Fiette1, Amina Sadik1, Heimanu Niebojewski1, Nils Rambal1, Maud Vinet3, Agnès Roussy2, Emmanuel Chanrion1, Benoit Bertrand1, and Pierre-André Mortemousque1.

1Univ. Grenoble Alpes, CEA, Leti, F-38000 Grenoble, France.

2SFL department, Centre Microélectronique de Provence, Ecole des Mines Saint-Etienne, Gardanne, France

3CEA Leti, now at Quobly, France

Significant progress has been achieved in automating the characterization of silicon Quantum Dot (QD) devices. Automated methods are implemented for large-scale characterization [1-3] in order to optimize devices geometry and manufacturing processes. However, the systematic large-scale study of QDs at the single-electron level remains challenging.

Here we introduce a semi-automatic method to characterize the single-electron regime of QDs within FD-SOI split-gate devices at 1K, based on a non-simplified version of the Constant Interaction (CI) model. Numerical resolution of CI model equations allows us to quantify the capacitances of a QD, in particular its mutual couplings with other QDs, and to extract its lever arms and charging energies.

This systematic characterization method has enabled us to carry out a large-scale comparative analysis on 300mm wafers. Firstly, we compare different device geometries with varying gate length and nanowire width and their effects on the mutual coupling between a QD in the few charges regime and its charge detector. Secondly, we demonstrate how tuning the back-gate polarization in FD-SOI devices can be used to systematically improve the mutual coupling, thereby optimizing charge readout.

[1] R. Pillarisetty et al. High Volume Electrical Characterization of Semiconductor Qubits (IEDM 2019).										                [2] L. Contamin, et al. Methodology for an efficient characterization flow of industrial grade Si-based qubit devices (IEDM 2022).								   [3] S. Neyens, et al. Probing single electrons across 300 mm spin qubit wafers (2023).