Optimizing Gate Control in Quantum Point Contacts and Quantum Dot Devices

Torbjørn Rasmussen1,4, Jacob Benestad2, Anton Zubchenko4, Danielle Middlebrooks6, Bertram Brovang1, Evert van Nieuwenburg3, Oswin Krause5, Justyna P. Zwolak6,7, Jeroen Danon2, Ferdinand Kuemmeth1, Anasua Chatterjee4

1Center for Quantum Devices, Niels Bohr Institute, University of Copenhagen, 2100 Copenhagen, Denmark

2Center for Quantum Spintronics, Department of Physics, Norwegian University of Science and Technology, NO-7491 Trondheim, Norway

3Lorentz Institute and Leiden Institute of Advanced Computer Science, Leiden University, P.O. Box 9506, 2300 RA Leiden, The Netherlands

4QuTech and Kavli Institute of Nanoscience, Delft University of Technology, 2600 GA Delft, The Netherlands

5Department of Computer Science, University of Copenhagen, 2100 Copenhagen,

Denmark

6National Institute of Standards and Technology, Gaithersburg, MD 20899, USA

7Joint Center for Quantum Information and Computer Science,

University of Maryland, College Park, MD 20742, USA

We present optimization strategies for controlling gate voltages in quantum devices, focusing on two experimental setups: a quantum point contact (QPC) with a 3x3 gate array and a semiconductor quantum dot (QD) system. In the QPC experiment, we employ the covariance matrix adaptation evolutionary strategy to optimize voltages, enhancing the conductance quantization by tuning the electrostatic potential in the transport channel. Simulations demonstrate the ability to offset random disorder potentials, with successful experimental validation showing improved conductance behavior.

In the QD experiment, we introduce a bootstrapping algorithm for initializing a depletion-mode QD device, enabling efficient scaling of qubit control systems. This protocol validates gate functionality, characterizes device parameters, and prepares a charge sensor for further tuning. The combined bootstrapping and coarse tuning modules successfully configure the device in under 8 minutes with a 96%success rate, setting a benchmark for performance and reliability in scalable quantum device operation.