# Optimising radio-frequency integration time for Hamiltonian estimation

Jacob Benestad1, Jan A. Krzywda2, Fabrizio Berritta3, Torbjørn Rasmussen3,4, Anasua Chatterjee4, Ferdinand Kuemmeth3, Evert van Nieuwenburg2, Oswin Krause3, Jeroen Danon1

1Center for Quantum Spintronics, Department of Physics, Norwegian University of Science and Technology, NO-7491 Trondheim, Norway

2Lorentz Institute and Leiden Institute of Advanced Computer Science, Leiden University, P.O. Box 9506, 2300 RA Leiden, The Netherlands

3Center for Quantum Devices, Niels Bohr Institute, University of Copenhagen, 2100 Copenhagen, Denmark

4QuTech and Kavli Institute of Nanoscience, Delft University of Technology, 2600 GA Delft, The Netherlands

We investigate how to find optimal strategies for choosing radio-frequency reflectometry integration times to balance the trade-off between smaller single-shot errors versus performing a bigger number of single-shot measurements, specifically in the context of Hamiltonian estimation experiments for spin qubits. Bayesian statistics is a powerful tool for such estimation based on readout measurements, typically after classifying based on a threshold, although a more complete method can easily be made by considering the raw data [1]. Building a statistical model for demodulated radio-frequency signal measurements, the pulse integration time can be incorporated into a Bayesian inference framework based either on thresholded binary measurements or the raw signal itself, from which an optimal value can be found numerically in simulations with analytical solutions available for posterior distributions approximised as Gaussians.

[1] Dinani et al. (2019), Phys. Rev. B 99, 125413