Zero-noise extrapolation-based quantum error mitigation in a silicon spin qubit

Hanseo Sohn1†, Jaewon Jung1†, Jaemin Park1†, Hyeongyu Jang1, Lucas E. A. Stehouwer2, Gavide Degli Esposti2, Giordano Scapucci2, and Dohun Kim1*

1Department of Physics and Astronomy, and Institute of Applied Physics, Seoul National University, Seoul 08826, Korea

2QuTech and Kavli Institute of Nanoscience, Delft University of Technology, PO Box 5046, 2600 GA Delft, The Netherlands

†These authors contributed equally to this work

*Corresponding author: dohunkim@snu.ac.kr

As quantum computing advances, error reduction remains crucial for near-term devices. Although quantum error correction-based fault-tolerant quantum computing offers ultimate error robustness, its successful implementation requires a large number of physical qubits with low average error rates, which are not yet widely applicable. Quantum error mitigation [1] provides a more immediate and resource-efficient alternative. Here, we present the implementation of a zero-noise extrapolation-based error mitigation technique [2,3] on a silicon spin qubit platform, building on successful demonstrations in other quantum systems such as superconducting qubits, trapped-ion qubits, and photonic processors. We first study three methods for noise amplification on a silicon spin qubit: global folding, local folding, and pulse stretching, using a standard randomized benchmarking protocol. We then employ global folding-based zero-noise extrapolation to the state tomography and improve the state fidelity from the unmitigated fidelities 75.82% and 82.16% to 99.96% and 98.52%, for different preparation states |-Y⟩ and |X⟩. The results demonstrate that zero-noise extrapolation is a versatile technique applicable across quantum computing platforms, regardless of their different noise characteristics via appropriate noise amplification methods.

Figure 1, Quantum state tomography results for two different prepared states: (a) |-Y⟩ and (b) |X⟩. (c) A schematic of the state tomography sequence for zero-noise extrapolation. (d) State fidelities of data with unmitigated (FRaw), with readout error mitigation (FREM only), and with both readout error mitigation and zero-noise extrapolation (FREM+ZNE).

[1] Z. Cai, R. Babbush, S. C. Benjamin, S. Endo, W. J. Huggins, Y. Li, J. R. McClean,

and T. E. O’Brien, Quantum Error Mitigation, Rev. Mod. Phys. 95, 045005 (2023).

[2] K. Temme, S. Bravyi, and J. M. Gambetta, Error Mitigation for Short-Depth

Quantum Circuits, Phys. Rev. Lett. 119, 180509 (2017).

[3] A. Kandala, K. Temme, A. D. Córcoles, A. Mezzacapo, J. M. Chow, and J. M. Gambetta, Error Mitigation Extends the Computational Reach of a Noisy Quantum Processor, Nature 567, 491 (2019)