# Dispersive readout of the donor-based flip-flop qubit

Si Yan Koh1, Mark Chiyuan Ma2, Hui Khoon Ng3,4, Teck Seng Koh1

1School of Physical and Mathematical Sciences, Nanyang Technological University, Singapore

2Department of Physics, National University of Singapore, Singapore

3Yale-NUS College, Singapore

4Centre for Quantum Technologies, National University of Singapore, Singapore

Superconducting resonators coupled to qubits offer a promising route to scalability, enabling both long-range operations and fast qubit readout. A key requirement is for qubits to have tunable electric dipole moments that couple strongly to the electric field of the resonator. The donor-based flip-flop qubit is a serious candidate due to the electron-nuclear spin states that can be controlled by microwave fields. However, a theoretical estimate of its spin-photon coupling strength is about 3 MHz [1, 2], placing it on the border between weak and strong coupling for typical planar resonator quality factors. Additionally, its lifetime is reduced by 8 orders of magnitude compared to donors in bulk silicon due to enhanced spin-valley relaxation from interface states [3]. It is therefore important to investigate whether the flip-flop qubit can leverage its advantages for dispersive readout. We show that good signal-to-noise ratios (SNR) and single-shot readout fidelities are possible across a range of qubit parameters. At most working points, the qubit remains sufficiently long lived to allow readout fidelities exceeding 90% in the weak coupling regime and fidelities > 99% in the strong coupling regime. Furthermore, such high fidelities are possible with fast readout times under 100 microseconds. Our results demonstrate that fast, high-fidelity dispersive readout of the flip-flop qubit using superconducting resonators is possible in both weak and strong coupling regimes.

[1] G. Tosi, F. A. Mohiyaddin, V. Schmitt et al. Nat Commun 8, 450 (2017).

[2] E. N. Osika, S. Kocsis, Y. Hsueh et al. Phys Rev Applied 17, 054007 (2022).

[3] P. Boross, G. Széchenyi, and A. Pályi. Nanotechnology 27, 314002 (2016).