# A singlet-triplet hole-spin qubit in planar silicon

Scott D. Liles1, Daniel J. Halverson1, Zhanning Wang1, Aaquib Shamim1, Rafael S. Eggli2, Ik Kyeong Jin1,3, Joe Hillier1, Krittika Kumar1, Isaac Vorreiter1, Matt Rendell1, Chris C. Escott4,5, Fay E. Hudson4,5, Wee Han Lim4,5, Dimi Culcer1, Andrew S. Dzurak4,5, and Alex R. Hamilton1

1School of Physics, University of New South Wales, Sydney, NSW, Australia.

2Department of Physics, University of Basel, Klingelbergstrasse 83, Basel, Switzerland.

3Center for Emergent Matter Science, RIKEN, 2-1 Hirosawa, Wako-shi, 351-0198, Saitama, Japan.

4Diraq, Sydney, NSW, 2052, Australia.

5School of Electrical Engineering and Telecommunications, University of New South Wales, Sydney, NSW, Australia.

Hole-spins in silicon quantum dots have been demonstrated to be highly promising for spin qubit applications. This is because holes experience a strong spin-orbit coupling, which enables rapid electrical manipulation, in-situ control over key parameters, and the ability to achieve sweet spots in the decoherence subspace. However, spin-orbit coupling also produces a complex response of the hole spin to the magnetic field orientation. This presents opportunities to optimize hole-spin qubits and deepen the understanding of spin-orbit effects in silicon.

In this work [1] we demonstrate a hole-spin singlet-triplet qubit formed using a planar MOS double quantum dot in silicon, with integrated SET charge-readout. Two axis control of the qubit is achieved with Rabi periods observed at 7ns with T2* of 300ns. We experimentally investigate the magnetic field anisotropy of the qubit eigenstates and relate the experimental results to a theoretical model of the system, which includes the effects of spin-orbit coupling and anisotropic site-dependent g-tensors. Comparison of the theoretical and experimental results provides insight into how spin-orbit coupling influences the hole-spin qubit operation. The demonstration of a high quality of the qubit (quality factor = T2*/TRabi >40) in a 2D planar architecture bodes well for scaling up to large arrays of coupled hole-spin qubits, or quantum circuit architectures.

Figure 1: a) 3D model of the planar silicon double quantum dot, which is used to form a singlet-triplet qubit. b-(bottom) The qubit is controlled by applying a manipulation pulse of duration S, which drives singlet-triplet oscillations. This experiment is performed for a range of in-plane magnetic fields (Bx). Ps is the measured singlet probability. b-(top) Shows a FFT of the singlet-triplet oscillations demonstrating coherent manipulation approaching 200MHz at 30mT.

[1] S. D. Liles et. al. Nature Communications 15, 7690 (2024).