# Universal control and benchmarking of four singlet-triplet qubits

Elizaveta Morozova1, 2, Xin Zhang1, 2, Maximilian Rimbach-Russ1, 2, Daniel Jirovec1, 2, Tzu-Kan Hsiao1, 2, Pablo Cova Farina1, 2, Chien-An Wang1, 2, Stefan D. Oosterhout1, 3, Amir Sammak1, 3, Giordano Scappucci1, 2, Menno Veldhorst1, 2, and Lieven M. K. Vandersypen1, 2

1QuTech, Delft University of Technology, Delft, The Netherlands

2Kavli Institute of Nanoscience, Delft University of Technology, Delft, The Netherlands

3Netherlands Organisation for Applied Scientific Research (TNO), Delft, The Netherlands

Semiconductor quantum dots have shown great promise for fault-tolerant digital quantum computation and analog quantum simulations. Significant progress has been made with individual spin qubits in both silicon [1] and germanium [2] in terms of scaling and performance. However, controlling large-scale arrays of individual spins presents multiple challenges, such as microwave heating and crosstalk. Unlike single-spin qubits, singlet-triplet qubits naturally align with the spin-readout basis in Pauli spin blockade and enable full electrical control using baseband voltages.

In this work, we investigate a 2x4 quantum dot ladder in Ge/SiGe, with controllable interactions between all neighboring spins, where we form four singlet-triplet qubits along the rungs of the ladder (Fig. 1 a). We demonstrate individual initialization, control, and readout of all qubits. We implement single-qubit gates along two axes of the Bloch sphere with average gate fidelities in the range of 99.49(8)-99.84(1)% obtained from randomized benchmarking (Fig. 1 b), and gate set tomography fidelities of 98.97(9)-99.61(5)%. Leveraging exchange interactions between qubits, we demonstrate a two-qubit SWAP-like gate with Bell-state fidelity of 73(1)-90(1)% and corresponding concurrences between 21(4)% and 64(4)% (Fig. 1 c). To further illustrate the remarkable level of control in the system, we create and distribute an entangled state across the array with remote Bell state fidelity of 75(2)% and concurrence of  22(4)%.

This work puts germanium singlet-triplet qubits firmly on the map as a potential platform for quantum computing.

Figure 1. a) A device scheme. b) Randomized benchmarking results for single-qubit gates, yielding average gate fidelity. c) Bell-state tomography fidelities for two-qubit gates and corresponding concurrences.

[1] Philips, S. G. et al. Universal control of a six-qubit quantum processor in silicon. Nature 609, 919–924 (2022).

[2] Wang C.-A. et al. Operating semiconductor quantum processors with hopping spins. arXiv:2402.18382 (2024).