Entangling electrons and nuclei in a four-qubit, two-atom device in silicon

Holly G. Stemp1,2, M. R. van Blankenstein1,2, A Vaartjes1,2, S. Asaad1,2, M. A. I. Johnson1,2, M. T. Madzik1,2, A. J. A. Heskes1,2, H. R. Firgau1,2, R. Y. Su1, C. H. Yang1, A. Laucht1,2,3, F. E. Hudson1,2,3, A. S. Dzurak1,2,3, K. M. Itoh4, A. M. Jakob2,5, B. C. Johnson6, D. N. Jamison2,5, A. Morello1,2

1School of Electrical Engineering and Telecommunications, UNSW Sydney, Sydney, New

South Wales 2052, Australia

2ARC Centre of Excellence for Quantum Computation and Communication Technology, Australia.

3Diraq Pty. Ltd., Sydney & 2052, Australia.

4School of Fundamental Science and Technology, Keio University, Minato City & 108-0073, Japan.

5School of Physics, University of Melbourne, Melbourne & 3010, Australia.

6School of Science, RMIT University, Melbourne & 3000, Australia.

Nuclear spins of donor atoms are candidate qubits that show very long coherence times but

remain challenging to couple. Here we present a four-qubit system consisting of two

phosphorus donor atoms in silicon, both with a coupled electron with spin degree of

freedom. The atoms couple over a distance of 15-22 nm through the exchange interaction of

the electrons. We use phase reversal tomography to show this coupling can be used to

entangle the electrons and nuclei. Benchmarking on the electrons has shown single qubit

fidelity of 99.9% and a Bell state fidelity of 93%[1]. The entangling gate for the nuclei is a

controlled-Z gate imposed by a geometric phase of one of the electrons. Comparing the

results to simulation, it seems the fidelity is severely limited by the electron initialization.

Where the procedure expects to load spin-down electrons onto the donor, spin-up electrons

are loaded an estimated 14% of the time. Efforts to reduce this error through a Maxwell’s

demon loading technique have been shown to be quite successful in earlier work[2].

[1] H. Stemp et al. (2023). Tomography of entangling two-qubit logic operations in exchange-coupled donor electron spin qubits (arXiv:2309.15463). arXiv. http://arxiv.org/abs/2309.15463

[2] M. A. Johnson et al. (2022) Beating the Thermal Limit of Qubit Initialization with a Bayesian Maxwell’s Demon. Phys. Rev. X 12, 041008