# Improvements towards hermetic packaging of levitating qubit devices

P. Russell1,2, K.L. Matkovic 1, A. Palmer1, L. Delventhal1, M.C. Cassidy1

1School of Physics, University of New South Wales

2School of Electrical Engineering, University of New South Wales

The advancement of levitating qubit devices presents a breakthrough in quantum computing [1]. Levitating electrons on noble gases allows the qubit to reside in vacuum, physically isolating itself from impurities and noise [2]. When combined with circuit quantum electrodynamics architecture, these platforms promise long coherence times and fast operation times, in a highly scalable manner [3]. Hermetic packaging plays a crucial role for these devices to maintain vacuum integrity. This allows us to encapsulate the noble gas [4] to allow for the isolation of qubits from environmental interactions.

Current setups are rudimentary and involve unreliable sealing methods, leaving large gaps for improvement. Figure 1 displays an example, where a seal must be made for each RF line and may leak either through the connector itself or the seal. As the number of control lines increases, the probability of having a hermetic cell rapidly diminishes.

We offer a more robust solution, which is more efficient to setup, and more scalable than existing packages. By using high density interconnects, and a multi-layer PCB, we greatly reduce the amount of indium seals required for ultra-high vacuum and minimize the footprint of the package.

Figure 2, Full section view of presented chip holder design.

Figure 1, Hermetic cell design taken from [3].

References

[1] A. Jennings, X. Zhou, I. Grytsenko, E. Kawakami; Quantum computing using floating electrons on cryogenic substrates: Potential and challenges. Appl. Phys. Lett. 18 March 2024; 124 (12): 120501. https://doi.org/10.1063/5.0179700

[2] Koolstra, G. et al. Trapping a single electron on superfluid helium using a superconducting resonator. Ph.D. thesis, The University of New Chicago (2019)

[3] Zhou, X., Koolstra, G., Zhang, X. et al. Single electrons on solid neon as a solid-state qubit platform. Nature 605, 46–50 (2022)

[4] Yang, G. Circuit Quantum Electrodynamics with electrons on helium. Ph.D. thesis, The University of New Chicago (2020).