# Spin Circuit QED in the Time Domain

X. Xue1, J. Dijkema1, P. Harvey-Collard1, T. Bonsen1, M. Rimbach-Russ1, G. Zheng1, S. de Snoo1, A. Sammak2, G. Scappuci1, L. M. K. Vandersypen1

1QuTech and Kavli Institute of Nanoscience, Delft University of Technology

2 QuTech and Netherlands Organization for Applied Scientific Research (TNO)

Semiconductor spin qubits hold promise for quantum computation due to their long coherence times and potential for scaling. So far, interactions between spin qubits are limited to spins a few hundreds of nanometers apart. A distributed architecture with local registers and long-range couplers will be needed to scale up to millions of qubits [1]. Circuit quantum electrodynamics can provide a pathway to realize interactions between distant spins [2].

In this poster, we present our realization of long-range spin-spin interactions using an on-chip superconducting resonator in two regimes. First with two spins detuned from the resonator frequency, allowing the demonstration of two-qubit iSWAP logic via virtual photons [3]. Next, in the resonant spin-photon regime, we use real resonator photons to show coherent spin-photon oscillations (vacuum Rabi oscillations) and utilize this to demonstrate spin-state transfer from the qubit at one end to the dot at the other end [4].

[1] M. A. Eriksson, L. M. K. Vandersypen. Quantum computing with semiconductor spins, Physics Today 72, 38–45 (2019)

[2] P. Harvey-Collard et al. Coherent Spin-Spin Coupling Mediated by Virtual Microwave Photons, Phys. Rev. X 12, 021026 (2022)

[3] J. Dijkema, X. Xue, et al. Two-qubit logic between distant spins in silicon, arXiv.org https://arxiv.org/abs/2310.16805v1 (2023)

[4] X. Xue, J. Dijkema, et al. in preparation