# Germanium Nuclear Spin Coupled to SiMOS Quantum Dots

Gauri Goenka1, Paul Steinacker1, Andrew Dzurak1,2, Andrea Morello1, Arne Laucht1,2

1 School Electrical Engineering and Telecommunications,

University of New South Wales, Sydney, NSW 2052, Australia

2 Diraq Pty. Ltd., Sydney, NSW, Australia

Background – With several contenders to realize qubits for quantum computation, SiMOS quantum dots are one of the leading qubit platforms. With their nanoscale size and CMOS compatibility, they possess the promise of being scaled to billions of physical qubits 1. Nuclear spins are often considered a noise source in SiMOS quantum dot qubit platforms, but they can also act as excellent quantum computation platform with extremely high coherence times at the order of many milliseconds 2. Studying nuclear spins, and in particular high-dimensional nuclear spins, as ancillas for quantum computation, will help scale down the size of quantum processors further and also pave the way forward for encoding error-corrected quantum information 3. Nuclear spin qubits/qudits (10-level systems) have been studied as donor-based platforms for quantum computation in silicon, but high-spin nuclei coupled to quantum dots remain understudied 3,4.

Objectives – To realize quantum computation based on a high spin nucleus, we aim to couple a single 73Ge nucleus to a SiMOS quantum dot, establishing initialization, readout, and coherent control of 73Ge, with the final goal to encode error-corrected quantum information into the 73Ge Hilbert space.

Methods – We have realized qubits at millikelvin and coupled a single quantum dot to a 73Ge nucleus.

Results & Conclusions – We recognize the presence of a single 73Ge spin coupling to a single quantum dot by monitoring the Electron Spin Resonance (ESR) frequency for a prolonged time. We detect 10 equally spaced resonance peaks (~300 kHz spacing), that correspond to the 10 levels of the 73Ge Hilbert space. The resolvable hyperfine coupling of ~300 kHz makes it promising to address the 73Ge Hilbert space, as being able to invert the electron spin at a particular frequency constitutes readout of the nuclear spin 2.

1.	Taylor, J. M. et al. Fault-tolerant architecture for quantum computation using electrically controlled semiconductor spins. Nat. Phys. 1, 177–183 (2005).

2.	Pla, J. J. et al. High-fidelity readout and control of a nuclear spin qubit in silicon. Nature 496, 334–338 (2013).

3.	Asaad, S. et al. Coherent electrical control of a single high-spin nucleus in silicon. Nature 579, 205–209 (2020).

4.	Fernandez De Fuentes, I. Magnetic control of Single 123Sb and 31P Implanted Donors in Silicon. (UNSW Sydney (main), 2023). doi:10.26190/unsworks/24834.